Emergency Response – Manure Application

In Case of a Spill, Leak, or Failure at the Storage Facility, During Transport, 
or Land Application, Immediately Implement the Following:

1.	Stop the source of the leak or spill.  For example:

- Turn off all pumps/valves and clamp hoses or park tractor on hoses to stop the flow of manure.

2.	Assess the situation and make appropriate calls for people, equipment, and materials.  See contacts below.

- Notify DNR spill hotline:  1-800-943-0003 (Spill reporting is mandatory by state law.)

- Call sheriff's office if spilled on public roads or its right-of-ways for traffic control.

- Clear the road and roadside of spilled material immediately.

3.	Contain the spill and prevent spillage from entering surface waters, tile intakes, or waterways.

- Use a skid loader or tractor with a blade to build dikes to contain or divert the spill or leak.

- Insert sleeves around tile intakes (or plug/cap intakes) and block down slope culverts.

- Use tillage implements to work up the ground ahead of the spill or use absorptive materials.

4.	Begin cleanup.

- Use pumps to recover liquids.

- Land apply on approved cropland at appropriate rates.

5.	Document your actions.

| Emergency Contacts | Contact Person (or Company) | Phone Number |
| --- | --- | --- |
| Fire/Rescue |  | 911 or 608 |
| County Sheriff |  | 911 or 608 |
| Farm Emergency Coordinator |  |  |
| DNR Hazardous Spill Line |  | 1-800-943-0003 |
| DNR WPDES Permit Contact |  |  |
| Available Equipment/Supplies | Contact Person (or Company) | Phone Number |
| On-farm Equipment Operator |  |  |
| Excavation Contractor |  |  |
| Manure Hauler |  |  |
| Septic Tank Pumping Truck |  |  |
| Veterinarian |  |  |
| Mortality Disposal Contractor | Darling International | 800-362-2917 |

Be prepared to provide the following information:

- Your name and contact information

- Farm address, location and other pertinent identification information.

- Nature of emergency (employee injury, fire, discharge of manure or hazardous materials).

- Emergency equipment and personnel that are needed.

- Potential for manure or hazardous materials to reach surface waters or major field drains.

- Current status of containment efforts.

- Location of hazardous/flammable materials and fire suppression equipment.

Site specific instructions: In the event of a manure spill or runoff event, contain by constructing a temporary berm using hay or straw bales found at the farm. Use soil from the most convenient location near spill. Protect nearby surface waters by plugging culverts with the hay or straw bales. Divert spill away from sensitive areas if needed. Skid loaders and other equipment can be found at the farm.